Resistor Notes									Score:  __________

Resistors job:

1. ____________________ the flow of current.

2. Can be used as a ________________ in a circuit.

3. Resistance is measured in __________________.

Two types of resistors:						Draw Symbol

4. ___________________			5.

6. ___________________			7.

Resistors makeup:

- Molded composition or __________________ resistor

  - Carbon material and a binder that holds the carbon together

- ____________ film

- _____________ wound

- Used when large amounts of _________ must be dissipated

11. The different sizes of carbon resistors tell the amount of ____________ ________________ that

a given resistor can transfer or dissipate to the outside air.

    - Size examples:  1/8, ¼, ½, 1 and 2 watts.

Resistor Color Code:				Black:  	0		B = ____________

Brown:	1		B = ____________

Red		2		R = ____________

Orange		3		O = ____________

Yellow		4		Y = ____________

Green		5		G = ____________

Blue		6		B = ____________

Violet		7		V = ____________

Gray		8		G = ____________

White		9		W = ____________

Tolerance Value:				Gold		5%

Silver		10%

No band	20%

How to read a fixed resistor:

- Each color represents a ______________.

- Position the resistor with the bands at your left.

- Make a list of colors and their respective numbers.

  - Band 1 =	Red =		_______

  - Band 2 =	Green = 	_______

  - Band 3 = 	Blue =	 	_______

  - Band 4 =	Gold = 	_______

- Write band 1 and band 2 numbers in order

  - 25

- Band 3 tells how many zero’s to add to band 1 and band 2

  - Blue tells us to add 6 zero’s

    - 25,000,000 or 25 million or 25 megohms. (used in electronics)

- Band 4 tells what tolerance the resistor has.

  - 22,500,000 minimal to maximum of 27.5 Megohms

Different ways resistance can be written:

- 25,000,000 ohms  =  25 megohms			*  25,000 ohms   =   25 K ohms

- 2,500,000 ohms  =  2.5 megohms			*  2,500 ohms	 =   2.5 K ohms

- 250,000 ohms	=  250 K ohms				*  250 ohms   =   250 ohms

Band 5:

A fifth band on a resistor is used to predict the percentage of failure per thousand hours of use.  The manufacturer conducts the test.

Brown - 1.0 %		Orange -   0.01%		Red      -   0.1 %		Yellow -   0.001%

What are the colors for?

25K Ohms								2.5K Ohms

Band 1:  __________							Band 1:  ___________

Band 2:  __________							Band 2:  ___________

Band 3:  __________							Band 3:  ___________

Variable Resistor:

Potentiometer: know as a “pot”

Measured in Ohms

10K ohm (example)

Value from 0 ohms – 10K ohms

AC = the measurement of the entire

Potentiometer (10K ohm “pot”)

  - AB + BC = AC (entire potentiometer)

Resistor Lab

Score:  ______________

Directions:  Record the color of the bands on each resister and calculate the value and tolerance.  Place your answers in the chart below.

| R# | First Color & Value | Second Color & Value | Third Color & Value | Tolerance Color & Value | Resister Value | Tolerance Minimum Maximum |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |

Potentiometers are variable resistors. 						Score:  ___________

|  | Directions: Turn the adjustment knob fully clockwise and measure the resistance values between all three leads. Repeat the process in the fully counter clockwise direction, and once again with the adjustment knob about in the middle. Record your measurements in the chart below. |
| --- | --- |

| Knob Direction | Leads Tested | Potentiometers |
| --- | --- | --- |
| Clockwise | AB BC AC |  |
| Counter-Clockwise | AB BC AC |  |
| Middle | AB BC AC |  |

- Why is the measurement CCW and CC different?

- Explain how a potentiometer works and what they are made of.

Test Your Knowledge							score:  ________________

- What is the basic unit of measure for capacitance?

- A potentiometer is another name for a:

  - Variable capacitor

  - Polarized capacitor

  - Fixed resistor

  - Variable resistor

- Place the number along side each color that corresponds to the color code found on a resistor.

  - White  __________

  - Orange  _____________

  - Gray  ____________

  - Red  ____________

  - Yellow  ____________

- The fifth band on a resistor is used to describe:

  - Failure rate

  - How much current it will carry

  - How long the resistor will last

- When putting a resistor in a circuit, the end with the colored bands is:

  - Connected to (-) side of battery

  - Connected to (+) side of battery

  - Connected to either side of battery

- A capacitor:

  - Increases the current

  - Is used to store a charge in a circuit

  - Decreases the resistance of a circuit

- The three factors that affect capacitance are:

  - Surface area, distance between plates, and insulating material

  - Size, length, and voltage of the circuit

  - Material, timing, and size

- Before you pick up any capacitor, you should:

  - Touch the two leads together

  - Discharge it

- If exact replacement capacitors are not available, you should substitute:

  - Pick a lower working voltage

  - Capacitor with the same rating, but higher working voltage

  - Keep shopping

- Name three requirements of a complete circuit.

- Define:

  - Dielectric:

  - Battery

  - Microfarad

  - Ohms

- A wire wound resistor should be used when:

  - You run out of the other types.

  - Only when you need a variable resistor

  - When large amounts of heat must be dissipated to the surrounding air