Intro to Metals

Situation Expectations

Situation 1:  Prepare to weld in the flat position.

PPE: Safety glasses, Welding helmet (minimum shade 10), Welding gloves, Leather Jacket

Area safety: free of flammable materials, fire extinguisher located, proper ventilation

Tools: Chipping Hammer, Welding tongs, Welding rods, Wire brush

Situation 2: Prepare to cut a piece of 3/8” Steel plate with the oxyfuel torch.

PPE: Safety glasses, Cutting goggles or face shield shade 5, Leather gloves, Leather Jacket

Area Safety: free of flammable materials, fire extinguisher located, proper ventilation

Tools: Oxyfuel cutting torch, Tip cleans, Flint striker

Situation 3: Prepare to move a full sheet of sheet metal through the shop.

PPE: Safety glasses, gloves

Area Safety: No obstructions, proper lifting techniques

Tools: None

Situation 4: Prepare to grind a piece of metal with a side grinder.

PPE: Safety glasses, Clear face shield, Ear plugs or muffs, Gloves

Area Safety: free of flammable materials, fire extinguisher located, appropriate ground fault circuit interrupter (GFCI)

Tools: Side grinder, Extension cord, C-clamps, Bench vice