DRILL PRESS SAFETY 				Handout & Note-Taking Sheet

SAFETY RULES:

- Make sure that the proper speed setting is set for your work—for holes greater than 1/2 inch, use lowest speed.

- Ensure that the bit is secure, to the full depth of the chuck, and centered when you mount it.

- Remove the key from the chuck after mounting the part.

- To prevent the drill bit from striking the table, position the feed stroke and table in a proper position.

- Clamp oddly shaped or small pieces to the table or secure them in a jig.

- When drilling holes completely through a work piece, a wood pad should be underneath it.

- Feed the drill bit slowly and withdraw frequently to rid shavings and expose the drill bit to air—the drill bit will overheat if this is not done!

- Always know where the power switch is.

SAFETY & DRILL PRESS PARTS:  While listening to your instructor, write the description in relation to safety for each part of the drill press below.

- On/Off Switch

- Column

- Table

- Index Pin

- Table Locking Clamp

- Universal Feed Lever

- Key Chuck

- Depth Stop

- Speed Adjustment

- Safety Guard

Created by Alison Burkett with information provided by John M. F. Berner