Mouse hole project

Name:					Date:

Layout Instructions:

- Cut 3 pieces of sheet metal . 6 3/8” X 3 1/4”

- Using a scribe, ruler, and divides layout the pieces of metal as shown in the drawing below.

- Do all necessary cutting using provided snips or notching equipment.

- Deburr all 3 piece of metal so that there are no sharp edges.

Bending Instructions:

- On the  ½” lines marked number 1 bend metal 180 degrees and flatten.

- On the  1” line marked number 2 bend metal 90 degrees.

- On the  1” line marked number 3 bend metal 90 degrees.

When this project is complete all 3 pieces of metal should be fabricated to the given measurements, free of any sharp edges and 3 piece should be identical.