One Piece Box Project

Name:				Date:

Layout Instructions:

- Cut 1 piece of sheet metal.  4 1/2” x  4 1/2”

- Using a scribe and ruler layout the piece of metal as shown on the drawing below.

- Do all necessary cutting using provided snips or notching equipment.

- Deburr the metal so that there are no sharp edges.

Bending Instructions:

- Bend all 1/4” edges 180 degrees and flatten.

- Bend all 1” lines up 90 degrees.

- Make sure your project is deburred and free of all sharp edges.