Sheet Metal Brake Information

Name:						Date:

A brake is a metalworking machine that causes sheet metal to bend.

Metalworking brakes consist of a flat surface onto which the material is placed, and a clamping bar which will come down and hold the material in place during the bending process. This clamping action can be manual, automatic or operated with a foot pedal. The front, gate-like, plate of the machine is hinged and may be lifted, forcing the material extended over a straight edge to bend to follow the plate.

The bends can be to any angle up to a practical limit of about 120 degrees, somewhat more in the case of a bar folder. If the area to be bent is narrow enough, a sharper bend (e.g., for a hem) can be made by inserting the bend under the clamping bar and lowering it.

Brakes come in sizes suitable for light aluminum or brass for small boxes and operated by hand, up to industrial sized and counterweighted hand operated or hydraulic machines suitable for large sheets of steel.

Four types of brakes are Cornice Brakes, Box-and-Pan Brakes, Bar Folders, and Press Brakes. A cornice brake only allows for simple bends and creases, while a box-and-pan brake also allows one to form box and pan shapes. It is also known as a bending machine or bending brake.

### Cornice Brake

A cornice brake has a solid clamping bar, the full width of the machine; thus, it can only make straight bends, the entire width of the work piece.

### Box-and-Pan Brake

In a box-and-pan brake, the clamping bar includes several removable blocks, which may be removed and rearranged to permit bending of restricted areas of a piece of sheet metal or of already partially formed pieces.

After bending, a box or pan form is then completed by screw, solder, weld, rivet, or other metal fixing process.

### Bar Folder

This is a simplified brake, usually much smaller than cornice or box-and-pan brakes. Typically, a single handle both clamps the work piece and makes the bend, in a single motion, but the depth is usually much less than what a cornice or box-and-pan brake can handle.

### Press brake

A more complex tool that forms predetermined bends by clamping the work piece between a matching punch and die.