Drill & Tap Project Handout

Name: __________________________________________________________ Date: _____________________________

- Saw:

Students will cut out 1 piece a metal using the saw provide by the instructor to 2 1/16” long by 2” wide by 1/2” 	think.

- File:

Students will file their piece of metal square where all side ar 90 degrees to each other to the finished 	measurements of 2” X 2” X 1/2”.

- Layout:

Students will use a combination square to check to make sure their project is square. Students will the use the 	combination square and a scribe to mark the center of their piece of metal.  Students will then use a center 	punch and hammer to indent the center of their piece of metal.

- Drill:

Students  will drill a hole in their piece a metal for a 1/2”  X 13 threads per inch tap. Students will refer to the 	drill tap chart for the proper size drill bit to drill their hole.

- Tap:

Students will tap their piece of metal with a 1/2” NC 13 tap.

- Deburr:

Students will deburr their piece of metal where it is free of all burrs and sharp edges.