# Metals Shop Safety Test

Answer Key

## Name: _______________________________Class: ______ Date: ____________

Arc Welding

1.  Is it safe to look at the welding arc with only sunglasses on? No

2.  Name 4 things you must wear while welding.

Welding hood

Gloves

Welding jacket

Safety glasses

3.  What is the hot metal picked up with? Tongs

4.  When you are tacking a piece of metal, do you need your welding hood down? Yes

5.  Name the 3 different polarities that is use in stick welding?

DC+

DC-

AC

6.  If you get cut or burnt, what should you do? Tell instructor

7.  The electrode holder holds the welding rod for welding.

8.  On an E7018 welding rod, what does the 70 stand for? Tensile strength

9.  How far off the metal should your hood be while welding? Reading distance

10.  There are 3 recommended lens shades of darkness for stick welding.  What are they?

10

11

12

11. What type shoes are recommended for welding? Leather high top boots

12. What type pants should you wear while welding? Denim 100% cotton

13. What is used to clean the metal before welding on it? Wire brush

Band Saw

1. How high off the material should the rollers be? 1/4”

2. What is used to push small pieces, to protect your fingers? Push block

3. How many teeth should be touching the material at one time? 2

4. What would cause the blade to bind? Too much pressure

5. What personal protection equipment should be used while operating the band saw? Safety glasses

Chop Saw

1. What personal protection equipment should be used? Face shield

2. Generally, which side of the saw should the longer material be? Left

3. What causes the blade to bind? Too much pressure

4. While cutting, how much pressure should be used to pull down the saw? Light pressure

5. Where must excess material be placed? Material rack

Hand Grinder

1. What should you not cover while grinding? Air vents

2. Where should the sparks be directed? Away from people and equipment

3. Where should the grinder be placed when finished? Where it belongs

4. How much pressure should be used while grinding? Light pressure

5. How should the grinder be placed when it is set down? Rock up

Torch

1. What is the maximum, safe, pressure for acetylene hose? 5 psi

2. A striker is use to light the torch.

3. How many turns is the acetylene bottle turned on? 1 1/2

4. How many turns is the oxygen bottle turned on. All the way

5. What type threads are the acetylene fittings? Left hand

6. While using a torch, a dark shield or goggles are use.  What shade is recommended? #5

7. List 4 personal protection equipment to be used while using a torch:

Gloves

Jacket

Cutting shield

Safety glasses

8. While cutting, where should the hose be located? Away from sparks

9. When you are finished using the torch, what should you do before leaving? Cut off

10. What is the hot metal picked up with? Tongs

Iron Worker

1.  When using the shear, where should the material be placed? Left side

2.  After cutting a short piece of metal, what must you check for? Small pieces of material still in machine

3.  The metal when punching a hole must be touching the guard/stripper when the punch is moving up.

4.  Who is responsible for safety around the iron worker? Operator

5.  Name 2 operations the iron work can perform?  punch	shear

Drill Press

1.  Where should the drill bit be placed after drilling? Drill index

2.  The stop that prevents the material from spinning is on which side of the part? Stop

3. If the part grabs and starts spinning, what is the 1st this you should do? Cut off machine

4.  Is it safe to wear gloves while using the drill press? No

5.  How much pressure should be used to drill? Light

Shop & Class Rules

List 3 class rules.

____________________________________________

____________________________________________

____________________________________________

List 3 shop safety rules.

____________________________________________

____________________________________________

____________________________________________