Lock Button Plate Quiz Answer Key

The industrial prints in this section will test your print reading ability.  Study the views, dimensions, title block, and notes in the diagram.  Read the quiz questions, refer to the print, and write your answers in the blanks provided.

- _Strip____ What style of title block is used on this print?

- __A______ What size drawing was used?

- _.074_____ List the decimal dimensions shown on the print that have 				unspecified tolerances.

- _44-66-0600 What is the part number?

- __Steel____ Name the kind of material used to make the part.

- __.150/.146  What limits are shown for the diameter of the two holes?

- _Engineering Change Order What does ECO mean?

- _Full____ To what scale was the drawing made?

- __R 1/16_ What radius is given for the four corners?

- _Remove all burrs What operation must be performed on the workpiece 	    	        according to the note on the print?

- _125_____What kind of finish is required on the workpiece?

- Hole size changed to .385/.380 What specific change resulted from revision B?

- .150, .385. .439, .877, .625, 1.250 List all the high limits shown on the print.

- _A331____ What is the drawing number?

- __+- .010__ What tolerance is used on the fractional dimensions?

- _Bilateral__ What specific kind of tolerance is dimension .876 called?

- _.437_____ What is the low limit of dimension .438?

- __+-.0005_ What tolerance is used for four place decimal dimensions?

- ___.615___ What is the low limit of the 5/8 fractional dimension?

- __1357____ The removal of a carburizing operation is specified by what ECO 		         number?