- Use of Bench and Pedestal Grinders

What safety procedures should you follow when using bench and pedestal grinders? - Fasten pedestal and bench grinders securely. - Ensure all the guards are in place and secure before using a grinder. - Adjust tool rests to within 3 mm (1/8 in.) of wheels. Never adjust rests while wheels are moving. Work rest height should be on horizontal centre line of the machine spindle. - Maintain 6 mm (1/4 in.) wheel exposure with a tongue guard or a movable guard. - Check that wheels have blotters on each side. - Check the wheel fits properly to the spindle when mounting. If it is loose, get another wheel. - Stand to one side of the grinder until the wheel reaches operating speed. - Bring work into contact with the grinding wheel slowly and smoothly, without bumping. - Apply gradual pressure to allow the wheel to warm up evenly. Use only the pressure required to complete a job. - Move the work back and forth across the face of the wheel. This movement prevents grooves from forming. - Wheels are made only for grinding certain items. Do not grind rough forgings on a small precision grinding wheel. - Dress wheels regularly. Do frequent, light dressings rather than one heavy dressings. - Support dressing tools so you can apply leverage without undue effort. With revolving cutter dressing tools use the lugs as anchors. - Replace worn wheels if you cannot dress it. - Ensure the grinder speed does not exceed the operating speed marked on the wheel. - Visually inspect wheels for possible damage before mounting. - Wear proper personal protective equipment: - eye, ear and face protection, - metatarsal safety boots, where required, - respiratory protection may be required, depending on the work. - Wear gloves only where necessary. What should you avoid when using bench and pedestal grinders? - Do not use a wheel that has been dropped. - Do not use a wheel that does not fit properly to the spindle. - Do not use excessive force to tighten the nut of the wheel. The force can crack the wheel. - Do not grind wood, plastics and non-iron metals on ordinary wheels. - Do not leave grinding wheels standing in liquids. The liquid can cause balance problems. - Do not grind on the side of a regular wheel..