Name: _______________________

Date: _______________

Test Attempt #: _______

Safety Test

1.  When the drill begins to break through the work, you should:

A.  ease up on the feed pressure.

B.  maintain the same feed pressure.

C.  stop the drill press immediately.

D.  apply more feed pressure.

2. If work is caught by the drill and starts to revolve, you should:

A.  stop the machine immediately.

B.  exert more feed pressure.

C.  decrease the feed pressure.

D.  grab the work with your hands.

3. Before operating the drill press you should:

A.  remove watches.

B.  remove rings.

C.  roll up long sleeves.

D.  all of the above.

4. Which of the following is a drill press safety rule?

A.  Increase the feed pressure when you break through the under-side of the work.

B.  Stand very close to the drill press.

C.  Never try to stop a revolving piece of work that is caught in the drill.

D.  All of the above.

5. Which of the following is a statement of safe practice?

A.  The smaller the hole, the slower the feed pressure.

B.  The larger the hole, the faster the speed.

C.  The smaller the hole, the slower the speed.

D.  The larger the hole, the slower the speed.

6. Drill press work should be held:

A.  by the hands.

B.  with a pair of tongs.

C.  in a vise or firmly clamped to the table.

D.  by another student.

7. The first thing you do before using the drill press is:

A.  remove the chuck key.

B.  put on your safety glasses.

C.  insert the chuck end of the drill bit.

D.  clean off the table.

8. The safest way to remove chips from the drill press table is with:

A.  a tilt of the table 45° to the left.

B.  a brush.

C.  a long ruler.

D.  your hands.

9. By removing the chuck key from the chuck before turning on the power, you will prevent the:

A.  chuck from being damaged.

B.  chuck from becoming unbalanced.

C.  chuck key from being thrown out at a high rate of speed.

D.  drill from breaking.

10. Touching the rotating chuck or drill with hands is:

A.  permissible if the instructor is watching.

B.  safe if a rag is used to protect the hands.

C.  done to slow down the rotating chuck.

D.  never permissible and very dangerous.