Calculating Feeds and Speeds	Name: 	_	_______________Date:

Cutting speed indicates the distance the work moves past the cutting tool. Feed is the distance the cutter moves lengthwise along the lathe bed during a single revolution.

Each manufacturer lists the appropriate speeds and feeds for their particular cutting tools. For rough cutting the typical feed rate is .010 to .020 (fpm).  Finish machining is typically .001 to .010 (fpm). These will vary depending on using high speed tools or carbide tools.

| In order to calculate feeds and speeds, you need RPM. Material Cutting Speeds (CS) |  |
| --- | --- |
| Aluminum & Aluminum Alloys | 200-300 |
| Cast Iron, soft | 100-150 |
| Cast Steel | 30-60 |
| Malleable Iron | 60-100 |
| Steel, Free Machining | 70-120 |
| Stainless Steel, Free Machining | 60-100 |
| Steel, Tool | 20-50 |
| Titanium | 15-20 |

The formula for calculating RPM is: RPM = (4 x CS) / D

  - D = diameter of work.

  - CS = cutting speed for material

Calculate RPM in the Following:

Problem #1:  Work diameter of 6” Aluminum. Feed rate .015 (fpm)

  - CS = 250

  - D = .750

  - RPM = (__ x 4) / ___

  - RPM = ____

Problem #2: Work diameter of 1 1/2” tool steel. Feed rate .005 (fpm)

- CS = 30

- D = .750

- RPM = (__ x 4) / ___

- RPM = ____

Problem #3: Work diameter of 3” malleable iron. Feed rate .020 (fpm)

- CS = 75

- D = .500

- RPM = (__ x 4) / ___

- RPM = ____

Problem #4: Work diameter of 4.75” titanium. Feed rate .010 (fpm)

- CS = 17.5

- D = .750

- RPM = (__ x 4) / ___

- RPM = ____

Determine the RPM for:

- Free machining stainless steel, work diameter of 3”. Feed rate .020 (fpm)

- Cast iron, work diameter of 4”.  Feed rate .015 (fpm)

- Aluminum, work diameter of 2 1/2”.  Feed rate .001 (fpm)

- Free machine steel, work diameter of 1 3/8”.  Feed rate .008 (fpm

- Titanium, work diameter of 1 ½”.  Feed rate .012 (fpm)