Post Test

Answer the following questions on the line to the right. (2 pts each)

- What is the formula used to find the circumference of a circle?			   C = 2πr___

- What other way can this formula be written?					   C = πd___

- What is the Pythagorean Theorem?						 a2 + b2 = c2_

- We can use the Pythagorean Theorem when

one angle of a triangle measures how many degrees?				____90°___

Solve the following problems. Show your work in the margins. (3 pts each)

- (1/4) x (7/12) = 7/48			3.   (1 ½) x (7/8) = 21/16

- 7/12 =	14/15				4.   11/16 = 11/2 =5 ½

5/8					         1/8

Measure the following lines using the 8th inch scale.  Write your answers on each line.

- 3.

Measure the following lines using the 16th inch scale.

- 4.

Instructor should measure these once printed to ensure consistency.

For each problem, show your work and include a diagram.  Circle the final answer. (up to 10 points each)

- A Right Triangle has a hypotenuse of an unknown length.  The other sides measure 72 cm and 54 cm.  What is the length of the hypotenuse?
(72)2 + (54)2 = c2

5184 + 2916 = c2

8100 = c2

c = 90 cm

- The longest side of a Right Triangle measures 20 in.  The shortest side measures 12 in.   What is the length of the remaining side?

c2 = a2 + b2

(20)2 = (12)2 + b2

400 = 144 + b2

b2 = 256

b = 16 in.

- A circle has a radius of 5/16 in.  What is the circumference of this circle?

c = 2r

c = 2(3.14)(5/16)

c = 1.96 in.

- A circle has a diameter that is 5 inches longer than its radius.  What is the circumference of this circle?

d = r + r		c = 2r

d=r+5 		c=2(3.14)(5)

r+5=r+r		c=31.4 in.

r=5

- Use the Pythagorean Theorem to find the length of side G.  Show your work to the right.

a2 + b2 = c2

32 + 82 = c2

9 + 64 = c2

73 = c2

c = 8.54 ”

Side G measures 8.54 ”

- Use the Pythagorean Theorem to find the length of side H.  Show your work to the right.

a=2  b=6-1=5  c=H

22  + 52 = c2

4 + 25 = c2

29 =c2

c= 5.39 ”

- Find the stretch out for a piece of duct work that is 6 ‘ long with a diameter of 12 ”.

Pay attention to units.

C = πd		or	C= πd

C = (3.14)(12”)		C = (3.14)(1 ’ )

C = 37.68 “ = 3.14 ’		C = 3.14 ’

The stretch out will be 6 ’ by 3.14 ’

- Find the circumference for a collar with the diameter of 6  “

C = πd

C = (3.14)(6 ”)

C = 18.84 ”

Bonus:  In the following diagram, combine what you know of the Pythagorean Theorem with what you have learned about Circumference to find the circumference of the circle (Hint: The Hypotenuse of the triangle goes through the middle of the circle.)

15 cm

20 cm

C = πd

d = c

a2 + b2 = c2

152 + 202 = c2

225 + 400 = c2

625 = c2

c = 25 cm = d

C = πd

C = (3.14)(25)

C = 78.5 cm