Oxy-Fuel Equipment Cutting Test Key

True- False

Directions:  Place a “T” in the blank if true; an “F” if false.  Make the false statements true by changing the underlined word.  Place answer  in the blank at the end of the statement.

_F___1.  The fuel gas most frequently used in welding is propane.   acetylene_____________________

_F___2.  The combination torch is used only for cutting.   Cutting, welding, heating________________

_T__3.  Most hand torches can cut metal up to seven inches thick.______________________________

_T__4.  It is possible to cut steel that is four inches thick.______________________________________

_T__5.  If acetylene is used in a tip with more than eight preheat holes or in a two-piece tip , the cutting                   tip will overheat and explode.____________________________________________________________

_F__6.  Acetylene should be used in a two-piece cutting tip. one piece__________________________

_T__7.  Acetylene cutting tips should be one piece with 1-6 preheat holes._________________________

_F__8.  The chance of flashback is less with low pressure regulator settings than with high pressure settings.  high pressure; low pressure___________________________________________________

_T__9.  Backfires and flashbacks are controlled the use of check valves and flashback

arrestors._____________________________________________________________________________

_F__10.  The oxy-fuel gas cutting torch works as a result of the metal being rapidly melted.   the flow rate of the gases______________________________________________________________________

_F__11.  Hard slag is easier to remove than soft slag.  harder___________________________________

_F__12.  Distortion is not a problem if the correct fue l gas is used.  amount of metal weld__________

13.  By looking at the drag lines in a cut, you can determine whether the cut was made correctly or incorrectly. 	a. True

14. The two major methods for controlling distortion while make a cut is making two parallel cuts at the same time and speed and making a short cut and skipping a strip to keep the plate from separating. a. True

15. The advantaged of a combination welding and cutting torch is it can be used for cutting, welding and heating. a. True

16. When cutting thick, round, dirty, or rusty, metals it is not necessary to use more preheat flames.      b. False

17. Oxygen is turned on while a welder is cleaning a tip to blow out debris. a. True

18. A welder can correctly set the gas pressure without a chart by watching the flame. a. True

19. If a metal is to be cut by oxyfuel gas cutting process the metal must have the ability to be oxidized.  a. True

Matching

Match the following words to the correct definition by placing the letter in the blank to the left of the word.

Words						Definitions

__G _20.  Oxide					A.  Pounds of pressure (gas)

__D_21.  Slag					B.  Temperature that a material will burn

__B_22.  Kindling temperature			C.  Mapp gas

__F_24.  Kerf					D.  Material left after a cut

_A_ 25.  Psig					E.  Hand cutting

_C__26.  MPS gases				F.  The width of cut a torch makes

G.  Rust