Lesson Plan # 1- Vocabulary Quiz Answer key

- S.M.A.W. Welding Machine- Equipment used to perform S.M.A.W. welding. This Machine transforms high voltage/low amperage electricity into low voltage /high amperage electricity suitable for welding.

- AC-Alternating Current

- Ground Clamp- Metal Clamp usually made of copper used to connect welding cable to part being welded.

- Wire Brush-typically wooden handled stiff bristled brush used to clean metal before and after welding.

- DCEP- Direct current electrode positive polarity. Commonly referred to as reverse polarity or DCRP (direct current reverse polarity) using older terminology.

- Electrode- in S.M.A.W. the electrode carries the arc and is consumed as welded material.

- Welding Cable- Flexible Copper stranded cable that is used to carry current to the electrode holder and to the ground cable.

- Polarity-Direction of current flow.

- DCEN-Direct Current Electrode Negative. Commonly referred to as straight polarity or DCSP (direct current straight polarity) using older terminology.

- Electrode Holder- The part of a welding outfit used to hold an electrode and conduct electricity to it.

- Chipping Hammer- Tool used to chip slag from a weld

- Amperage-This is a measurement of the amount of electricity flowing in a circuit.