| SMAW Weld Beads |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Unacceptable 0 pts | Needs Improvement 2 pts | Acceptable 4 pts | Very Good 5 pts |  |
| Safety Practices | Unacceptable No welding helmet worn. No safety glasses worn. Student will not be allowed to continue the assignment. | Needs Improvement Two or more of the following safety practices violated: No welding gloves worn. Incorrect shoes worn. Incorrect clothing worn. Hot metal carried in gloved hand(s). Did not use proper eye protection. | Acceptable One or more of the following safety practices violated: No welding gloves worn. Incorrect shoes or clothing worn. Hot metal carried in gloved hands. Did not use proper eye protection. | Very Good All of the following safety practices were followed: Proper welding gloves worn. Proper shoes worn. Proper clothing worn. Pliers, etc. used to carry hot metal. Proper eye protection was used. |  |
| Bead Width | Unacceptable Bead unsuccessfully made. | Needs Improvement Bead narrower or wider than width of two electrodes used. | Acceptable The majority of the bead is equal to the width of two electrodes used. | Very Good Bead width equals the width of two electrodes used. |  |
| Bead Shape | Unacceptable Bead unsuccessfully made. | Needs Improvement Bead does not form the acceptable shape - has one of following problems. Too flat Too high and does not blend into the base metal at the weld toes. | Acceptable Proper bead shape is not consistent for the entire length of the bead. | Very Good Bead has an acceptable shape for the entire weld. |  |
| Weld Defects | Unacceptable Bead unsuccessfully made. | Needs Improvement More than two of the following defects are present: Excess spatter is present. Bead is not uniform. Weld ripples are long and pointed. | Acceptable 1 of the following defects are present: Excess spatter is present. Bead is not uniform. Weld ripples are long and pointed. | Very Good No weld defects are present. |  |
| Start / Finish of Weld | Unacceptable Bead unsuccessfully made. | Needs Improvement Start of weld (width) is not the same as the finish. Crater not filled in. | Acceptable Start of weld (width) is not the same as the finish. Crater not filled in. | Very Good Start of weld same width as finish. Crater filled in. |  |

L

Used with permission from Miller Electric