PVC Plumbing Project

45

90

Male Adaptor

Spigot

11 ½”

8”

11”

45

Thread Tee

14”

10 ½”

Slip Tee

16”

4”

90

Instructions: Make this configuration by following the directions below out of PVC pipe using the different fittings you learned.

How to assemble the PVC pipe:

- Cut the pipe the lengths needed to construct the configuration shown above.

- Clean the end of the pipe of ragged edges using sand paper.

- Also clean the inside of the fitting to remove any dirt or grease.

- Apply the pipe solvent around the end of the pipe and also the inside of the fitting.

- Push the fitting and the pipe together. Twist the pipe to distribute the solvent evenly in the joint.

- Keep pressure on the pipe and fitting for at least 30 seconds to prevent the pipe from backing out of the fitting.

- Repeat until all fitting have been attached

- Remove the excess glue with a clean rag.

- Let the glue cure for at least 15 minutes before handling.

- To attach spigot apply Teflon tape around the male end of the spigot.

- Twist the spigot into the Threaded Tee for a tight fit.

To Test: Under teacher supervision, take finished plumbing project outside and hook up to the water hose to see if it leaks.

Bill of Materials

Name: ______________________			Date: ____________________

Company Name ________________________________

Pipe Used 	_________ Ft 		x	$____ per foot 	=      ____________

90 Elbow 	# used ________ 	x 	$_____ each 		=      ____________

45 Elbow 	# used ________ 	x 	$_____ each 		=      ____________

Slip Tee 	# used ________ 	x 	$_____ each 		=      ____________

Threaded Tee 	# used ________ 	x 	$_____ each 		=      ____________

Spigot 		# used ________	x 	$ _____each 		=      ____________

Adaptor  	# used ________ 	x 	$_____ each 		=      ____________

Teflon Tape 	# used ________ 	x 	$_____ each 		=      ____________

Glue 	 	# used ________ 	x 	$_____ each 		=      ____________

Subtotal = _____________

Labor 		$_____ per hour 	x 	______ hours 		=     _____________

Total    =     _____________

Pipe can be calculated by added all the inches together (4, 8, 10 ½, 11, 11 ½, 14, & 16)

Divide by 12 (there are 12 inches in 1 foot)

Round up to the nearest whole  number