Fluid Power Mathematics Worksheet

Area of a Circle: A  =  π r2  	(use π = 3.14)

Pressure: P =  F			Force:  F = P x A			Area:  A =  F

A								     P

- If an actuator’s piston has a diameter of 5 inches, what is its radius?

- If the actuator’s piston has a radius that is 2 inches, what is its area?

- If the actuator’s piston has a diameter that is 3 inches, what is its area?

- What is the pressure in pounds per square inch (PSI) if 100 pounds of force are applied to a hydraulic system’s piston with an area of 2 inches?

- What is the pressure in pounds per square inch (PSI) if 50 pounds of force are applied to a hydraulic system’s piston with an area of 4 inches?

- What is the force of an actuator with a piston that has an area of 3 square inches ( in2 ) and a 50 psi?

- What is the force of an actuator with a piston that has an area of 5 square inches ( in2 ) and a 75 psi?

- If we want 50 pounds of force and we know the pressure is 10 psi, how much area does the piston in the actuator need?

- If we need a piston with an area of 314 inches, what would its radius need to be?

- If we have two actuators connected together and one actuator has 50lbs of force applied to its piston that has an area of 2 square inches, how big area must the other actuator have to produce 500 lbs of force?

- 2.5 in.

- 12.57 in2

- 7.07 in2

- 50 PSI

- 12.5 PSI

- 150 lbs

- 375 lbs

- 5 in2

- 10 in

- First we must determine the pressure, using the information we know about the first actuator.

F = 50, A = 2

50 = P * 2

P = 25 psi

Now we can determine the Area of the second actuator using that we know the force of the second actuator has to be (500-50) = 450 lbs.

F = 450, P = 25

450 = 25 * A

A = 18 square inches